SECTION 08 71 91

BARN DOOR HARDWARE

GENERAL

SECTION INCLUDES

Provide track and hardware for sliding door systems, including:

Enclosed barn door hardware.

RELATED SECTIONS

Section 06 10 00 – Rough Carpentry.

SUBMITTALS

Submit under provisions of Section 01 30 00.

Product Data:

Manufacturer’s data sheets and instructions.

Storage requirements and handling recommendations.

Shop Drawings:  Submit details of construction and adjacent structures. Provide reference measurements in compliance with manufacturer’s specifications.

Closeout Submittals:

Operation and maintenance data.

Provide any specific tools required for owner’s adjustments.

QUALITY ASSURANCE

Manufacturer:  Minimum of 5 years of experience manufacturing similar products.

Installer:  Minimum of 2 years of experience installing similar products.

Preparation: Coordinate preparation of adjacent construction and field measurements to ensure proper support and installation.

DELIVERY, STORAGE AND HANDLING

Products and materials are to be delivered in unopened factory labeled packages. Accessories are to be delivered together with door hardware. Store and handle according to manufacturer's instructions and recommendations and with obedience to general best practices.

PRODUCTS

MANUFACTURER

FALKENSTEIN (dba, Bartels USA, Inc.), 1901 Tigertail Blvd, FL-33004, 786-791-8557, sales@falkenstein-doors.com, falkenstein-doors.com

No substitutions permitted.

Requests for substitutions will be considered in accordance with provisions of Section 01 60 00 - Product Requirements.

BARN DOOR HARDWARE

Barn Door Track: BOXtrack with the following characteristics:

Enclosed barn door hardware including end caps with wood or frameless glass mounting hardware

Length: 78 ¾” / custom up to 19’ 8 ¼”

Material: Extruded aluminum, stainless steel look or black anodized finish.

Track: 2-9/16” (64.5 mm) x 2-3/8” (60 mm)

Door width: Variable, min. 21 ⅝” (550 mm)

Includes ball bearing carriages with a capacity of 265 lb (120 kg) tested for a min. of 100,000 cycles at full capacity

Door thickness:

Wood doors: 1- ⅛ to 2 inches (35 to 51 mm)

Frameless glass doors: ⅜“ (10 mm), ½” (12.7 mm) or ⁹⁄₁₆” (13.5 mm)

With hold-in-place stoppers or double Soft-Close (min. 25 ⁹⁄₁₆” door width, max. 176 lb door weight)

Bi-parting applications available with door synchronization

Optional distance profile with matching finish to increase wall clearance by ¾” (20 mm)

Anti-skid screw to prevent jumping of wheels

| Acceptable mfg.: |  | Falkenstein NA |
| --- | --- | --- |
| a. | Enclosed barn door track for wood and frameless glass –265 lb | BOXtrack |

EXECUTION

EXAMINATION

Be sure that adjacent and supporting structures are constructed to bear the load of the system and door to be installed.

Examine substrates for compliance with requirements for installation tolerances and other conditions affecting performance.  Proceed with installation only after unsatisfactory conditions have been corrected.

PREPARATION

Clean surfaces thoroughly prior to installation.

If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

INSTALLATION

Install products in strict accordance with manufacturer’s instructions, approved submittals and in proper relationship with adjacent construction.

Install hardware level, plumb, sure, true and straight.

Clean track and hardware surfaces before hanging sliding doors.

Test for proper operation and adjust until satisfactory results are obtained. Replace units which do not operate properly.

Protection

Protect installed products until completion of project.

Clean surfaces to remove soiling, stains, dust, and dirt using materials acceptable to manufacturer.

END OF SECTION